# VIIRS Terrestrial Evapotranspiration (ET) Data Products (VNP16/VJ116) User Guide

Maosheng Zhao1,2, Qiaozhen Mu1,2, Arthur Endsley3, John Kimball3, Sadashiva Devadiga2, Steven Running3

1SSAI, 2NASA GSFC, 3NTSG, University of Montana

Version 1.0, October 2024

Table of Contents

Preface	1

1. Synopsis	1

2. Background and Algorithm Overview	2

2.1 Surface Energy Balance	2

2.2 The Penman-Monteith (PM) Approach	3

3. Evapotranspiration Model Logic	4

3.1 The VNP16 Algorithm	4

3.1.2 Vapor Pressure, Humidity, and Saturated Fraction	6

3.1.3 Evaporation from Wet Canopy	7

3.1.4 Evaporation from Bare Soil Surfaces	8

3.1.5 Canopy Transpiration	10

3.2 Total Daily ET and Potential ET	11

3.3 Free Parameters	12

4. Operational Details and Uncertainty of VNP16	12

4.1 Dependence on MODIS Land-Cover Classification	12

4.2 Assumption of Constant Biome Properties	14

4.3 Vegetation Properties and Albedo	16

4.4 Cloud/Aerosol Screening and Year-End Gap-Filling	16

4.4.1 fPAR/LAI Climatology and Forward VNP16A2	17

4.4.2 Year-end Gap-filled VNP16A2[3]GF	19

4.5 GMAO Hourly Meteorological Data	21

4.6 Validation of VNP16/VJ116	23

5. Description of 500-m VNP16/VJ116 Data Products	24

5.1 File Naming Conventions	24

5.2 Downloading VNP16 Datasets	25

5.3 Description of VNP16 Datasets	25

5.3.1 8-day VNP16A2 (or VNP16A2GF)	25

5.3.2 Annual VNP16A3GF/VJ116A3GF	27

6. List of NTSG Co-Authored Papers Using MOD16	29

7. References	32

# Preface

The operational VIIRS global Collection 2 (C2) terrestrial evapotranspiration (ET) and its potential (PET) data products, their forward 8-day VNP16A2/VJ116A2 and year-end Gap-Filled (GF) 8-day VNP16A2GF/VJ116A2GF and annual VNP16A3GF/VJ116A3GF use a newly recalibrated BPLUT (Table 4.2) for biophysical parameters (Endsley et al., 2024) and a recent version of NASA’s GMAO meteorological data: Goddard Earth Observing System for Instrument Teams (GEOS-IT). In contrast, the Collection 6.1 (C6.1) MODIS is still using the old Collection 6 (C6) or C5 BPLUT and the old version of GMAO data: GEOS for Forward Processing for Instrument Teams (GEOS-FP-IT).  As a result, these differences cause inconsistency between the C2 VIIRS ET/PET and C6.1 MODIS ET/PET (8-day MOD16A2/MYD16A2, MOD16A2GF/MYD16A2GF and annual MOD16A3GF/MYD16A3GF).  MODIS Collection 7 (C7) ET/PET will use the newly recalibrated BPLUT and the same NASA’s GMAO global meteorological data, GEOS-IT, to be consistent with the C2 VIIRS ones.

# 1. Synopsis

Terrestrial evapotranspiration (ET) represents the primary loss of water from soil and vegetation to the atmosphere, and together with its latent energy (LE) counterpart, provides the primary link between the global water and energy cycles. Thus ET is a sensitive environmental indicator of land surface and vegetation conditions, and their response to changing climate.

This guide provides a description of the Collection 2 (C2 hereafter) NASA VIIRS level-4 terrestrial ecosystem Evapotranspiration (ET), latent heat flux (LE), Potential ET (PET), and Potential LE (PLE) data products derived at 500m spatial resolution, including: the 8-day forward VNP16A2/VJ116A2 record and the year-end gap-filled 8-day VNP16A2GF/VJ116A2GF and annual VNP16A3GF/VJ116A3GF data products. The operational VIIRS ET data products provide for continuity beyond the NASA EOS Terra/Aqua MODIS heritage ET data products: MOD16 and MYD16 (Mu et al., 2007; Mu et al., 2011). The two short names of VNP and VJ1 refer to the VIIRS sensors onboard the respective Suomi-NPP (SNPP) and JPSS-1 satellites. The GF label refers to Gap-Filled following the procedure summarized below. The content and format of the VNP16 and VJ116 data products are largely consistent with their MODIS heritage products, except for minor differences due to the different satellite platforms and sensors.

Cloud contamination is a major issue for NASA’s forward operational land data products because it reduces the usefulness of the remotely sensed data for land information. To resolve potential negative impacts from cloud contamination and missing data in the time series of remotely sensed inputs to VIIRS ET, at the end of each year, when a full data record of all VIIRS 8-day Fraction of Absorbed Photosynthetically active Radiation (fPAR) and Leaf Area Index (LAI) from VNP15A2H or VJ115A2H are available, NASA reprocesses the year-end Gap Filled (GF) VIIRS ET/PET/LE/PLE series following a method originally developed for the MODIS GPP and NPP GF data products (Zhao et al., 2005; Mu et al., 2007). To distinguish from the forward data products, The GF data products are named as 8-day VNP16A2GF, VJ116A2GF and annual VNP16A3GF and VJ116A3GF.

The VIIRS ET algorithm is based on the logic of the Penman-Monteith equation, which uses daily meteorological reanalysis data and remotely sensed daily surface albedo (from 16-day rolling VIIRS BRDF VNP43MA3 and VJ143MA3) and 8-day vegetation property dynamics in terms of fPAR/LAI from the VIIRS sensor as inputs. The resulting data products are archived and distributed by NASA and the USGS LP-DAAC (Distributed Active Archive Center). This document is intended to provide a detailed overview of the ET algorithm and product structure to allow for the successful use of the data in research and applications.

Since the content of the VNP16 and VJ116 data products are mostly the same, except for minor differences mainly caused by the different satellite platforms, we use VNP16 hereafter to refer to both VNP16 and VJ116 across this document; VJ116 is referenced only for specific cases. This User Guide (UG) is the revised and improved version of the previous UG, for the Collection 6.1 MODIS ET and PET product version. It has been specifically updated to reflect the VIIRS ET and PET data products.

# 2. Background and Algorithm Overview

Calculation of ET is typically based on the conservation of either energy or mass, or both. Computing ET is a combination of two complex components occurring within a given grid cell and time step: (1) estimating the stomatal conductance to derive transpiration from vegetation cover; and (2) estimating evaporation from the wet canopy (when present) and ground surface. The VNP16 ET algorithm runs on a daily (24-hr) time step, represented as the sum of ET from day- and nighttime. Vertically, ET is the sum of water vapor fluxes from soil evaporation, wet canopy evaporation and plant transpiration from the dry canopy surface. Remote sensing has long been recognized as the most feasible means to provide spatially distributed regional ET information on land surfaces. Remotely sensed data, especially those from polar-orbiting satellites, provide temporally and spatially continuous information over vegetated surfaces useful for regional measurement and monitoring of surface biophysical variables affecting ET, including albedo, biome type, and leaf area index (Los et al., 2000).

## 2.1 Surface Energy Balance

Net radiation intercepted by the earth’s surface, R, can be partitioned into fluxes of sensible heat (H), latent heat (λE), ground heat (G), and the change in heat storage (ΔS):

R=H+λE+G+ΔS								(2.1)

Latent heat, or heat that has been used to vaporize water, is the quantity of interest in the VNP16 algorithm and it can generally be described in terms:

λE=ρCpγesat-erA+rS									(2.2)

The individual terms are described in the sections below, but the general idea is that the latent heat flux:

- Increases with the air’s capacity to store water vapor, esat-e ;

- Increases with the heat storage capacity of air, ρCp ;

- Decreases with increasing aerodynamic and surface resistances, rA + rS .

The flux of latent heat is also termed evapotranspiration (ET) because it includes both evaporated water and transpired water vapor fluxes. Latent heat and ET are related through the latent heat of vaporization (λ), which is the energy required to transform water from liquid to gas phase, providing a critical link between the energy and water cycles. Most discussions of modeling ET begin with a version of the equation above, which gets complicated quickly when we try to calculate ET over large areas using weather data. As such, our description of the VNP16 ET model is instead procedural, to aid implementation.

## 2.2 The Penman-Monteith (PM) Approach

Developing a robust algorithm to estimate global evapotranspiration is a significant challenge. Traditional energy balance models of ET require explicit characterization of numerous physical parameters, many of which are difficult to determine globally. For these models, thermal remote sensing data (e.g., land surface temperature, LST) are the most important inputs. However, using the 8-day composite MODIS LST (the average LST of all cloud-free data in the compositing window) (Wan et al., 2002) and daily meteorological data recorded at the flux tower, Cleugh et al. (2007) demonstrated that the results from thermal models can be unreliable. Using a combination of remote sensing and global meteorological data, we adapted the Cleugh et al. (2007) algorithm, which is based on the Penman–Monteith (PM) method and calculates both canopy conductance and ET. Monteith (1965) eliminated surface temperature from Equations 2.1 and 2.2 to give the following form:

λE=sA+ρCpesat-erA-1s+γ1+rSrA-1								(2.3)

where s=desatdt, the slope of the curve relating saturated water vapor pressure (SVP or esat) to temperature; A is available energy partitioned between sensible heat and latent heat fluxes on land surface. VPD=esat-e is the air vapor pressure deficit, the difference between SVP and the actual vapor pressure (AVP or e). All inputs have been previously defined except for surface resistance, rS, which is an effective resistance accounting for evaporation from the soil surface and transpiration from the plant canopy.

Despite its theoretical appeal, the routine implementation of the PM equation is often hindered by requiring meteorological forcing data (A, air temperature, and VPD) and the aerodynamic and surface resistances (rA and rS). Radiation and soil heat flux measurements are needed to determine A; air temperature and humidity to calculate VPD; and wind speed and surface roughness parameters to determine rA. Multi-temporal implementation of the PM model at regional scales therefore requires routine surface meteorological observations of air temperature, humidity, solar radiation, and wind speed. Models for estimating maximum stomatal conductance, including the effect of limited soil water availability and stomatal physiology, requires either a fully coupled biophysical model such as that by Tuzet et al. (2003) or resorting to the empirical discount functions of Jarvis (1976), which must be calibrated. Determining a surface resistance for partial canopy cover is even more challenging with various dual source models proposed (e.g., Shuttleworth and Wallace, 1985) to account for the presence of plants and soil.

# 3. Evapotranspiration Model Logic

## 3.1 The VNP16 Algorithm

The evaporation of water is like a commercial transaction in which a wet surface sells water vapour to its environment in exchange for heat…The environment can supply heat by solar radiation, by turbulent transfer from the atmosphere, or by conduction from the soil. - Monteith (1965)

VNP16 is based on the Penman-Monteith (PM) approach to calculating evapotranspiration (ET). The central idea of the PM approach is that it combines energy conservation and the saturation vapor pressure of the air to determine: 1) How much energy is available to vaporize water; and 2) When to stop vaporizing water (when the air is saturated). Figure 1 shows the logic behind the VNP16 algorithm for calculating daily ET.

Figure 1: Flowchart of the VNP16 ET algorithm (from the MODIS ET algorithm in Mu et al., 2011). LAI: leaf area index; fPAR: Fraction of Photosynthetically Active Radiation.

ET is the sum of three components (three sources of latent heat): transpiration, evaporation from wet canopy surfaces, and evaporation from bare soil surfaces. The VNP16 algorithm calculates separate daytime (i.e., sun in the sky) and nighttime quantities for each of these components for each daily (24-hr) time step. We can understand the VNP16 algorithm as a series of steps to calculate each component wherein the input drivers are partitioned into daytime and nighttime values, starting with the calculation of the net radiation received by a surface. Throughout, unless otherwise specified: temperature is given in degrees Kelvin; pressure is given in Pascals (Pa); resistance is given in seconds per meter (s m-1). “iff” means “if and only if.”

3.1.1 Net Radiation Available

The net radiation available to vaporize water, A, is the sum of down-welling, short-wave radiation intercepted from the sun, RS↓, and net long-wave radiation (LWGNT), RL, from the earth and atmosphere:

A=1-αRS↓+RL								(3.1)

where α is the short-wave albedo (under “black-sky” conditions), i.e., the fraction of down-welling, short-wave radiation that is not absorbed by the earth’s surface. MOD16 (precursor to VNP16) used to calculate the latter term using the emissivity of the earth’s surface; as of MODIS Collection 6.1, we instead directly use surface net downward longwave flux (LWGNT) from the global meteorological data provided by GMAO/NASA. Because the sun is the only source of short-wave radiation in this system, at nighttime, RS↓=0 and the only source of energy is RL.

The three components of ET imply there are two surface sources of water vapor: the vegetation canopy (leaf surfaces) and bare soil. Therefore, the next step is to calculate the net radiation received by each of these surfaces. The net radiation received by the soil is a balance between incoming radiation, A, and the ground heat flux, G, modulated by the fraction of vegetation cover, FC; i.e., if the ground is completed covered by vegetation (FC=1.0), then there is no ground heat flux. In VNP16, we use the fraction of photosynthetically active radiation (fPAR), a measure of the fraction of ground area covered by photosynthetic vegetation, as our measure of FC (Los et al., 2000). Therefore, net radiation received by the soil surface is calculated as:

Asoil=1-FC×A-G							(3.2)

Ground heat flux is calculated based on the surface energy balance and a pre-determined (calibrated) minimum temperature constraint (in Celsius), Tmin,close, which represents the temperature below which plant stomata are assumed to be completely closed. If the mean annual temperature is between Tmin,close and 25 degrees C, and if the contrast between daytime and nighttime air temperatures is greater than 5 degrees C, then ground heat flux is calculated based on an empirical relationship determined using field data and spectral vegetation indices (Jacobsen and Hansen 1999; Mu et al. 2011):

G=4.73T-20.87           iff   Tmin,close≤Tannual<25,Tday-Tnight≥5        (3.3)

Otherwise, ground heat flux is zero. When non-zero (as above), we additionally constrain G so that it is no greater than 39% of the net radiation to the land surface:

G=0.39A   iff   G>0.39A     						(3.4)

Again, G and Asoil are calculated separately for daytime and nighttime input values. When calculating daytime G, if the daytime G would be greater than A, then G=A. When calculating nighttime G, if nighttime A (equivalent to RL) minus G would be less than -0.5 times daytime A, then G is taken to be equal to nighttime A plus half of daytime A.

### 3.1.2 Vapor Pressure, Humidity, and Saturated Fraction

There are a number of quantities used in calculating all three components that must be calculated separately for daytime and nighttime inputs.

The saturation vapor pressure (SVP) in Pascal is calculated using the below equation (Abbott and Tabony, 1985; the formular was cited by Alduchov and Eskridge, 1996):

SVP=610.7 ×exp⁡(17.38 × (T-273.15)239 +(T-273.15)) 			(3.5)

Where T is the temperature in Kelvin.

Vapor pressure deficit (VPD) can be calculated a variety of ways but is always defined as the difference between SVP and the actual vapor pressure (AVP); hence, VNP16 calculates VPD by first calculating the AVP, after Gates (1980):

AVP=QV10M×P0.622+0.379×QV10M							(3.6)

where QV10M is the water vapor mixing ratio at 10-meter height (units: kg/kg) and P is the surface pressure (units: Pa) (Note that here P is from the global meteorological data GMAO/NASA not derived from elevation.  For all the rest air pressures, they are derived from elevation using the Equation 3.14). VPD (units: Pa) is the differences between SVP and AVP:

VPD=SVP-AVP

VPD=0 iff VPD<0								(3.7)

Note that T is day and nighttime dependent to distinguish day/nighttime SVP and VPD.

Relative humidity (RH) can then be calculated as the percentage of the ratio of AVP to SVP:

RH=100×AVPSVP

RH = 100  iff  RH > 100								(3.8)

After Fisher et al. (2008), we calculate the fraction of the land surface that is saturated, Fwet, based on the relative humidity:

Fwet=0if RH<70%(RH/100)4otherwise				                        (3.9)

The latent heat of vaporization, λ, (units: Joules per kilogram) is a key term that quantifies the amount of energy required to vaporize a kilogram of water, based on the current temperature (units: Kelvin) of the water:

λT=2.501-0.002361×T-273.15×106				(3.10)

The psychrometric constant, which relates the vapor pressure of air to its temperature, is calculated as:

γ=Cp×Pλ×0.622									(3.11)

This is described by Maidment (1969). Cp=1013 (J per kilogram per Kelvin) is the specific heat capacity of air, P is the air pressure, and 0.622 is the ratio of molecular weights, water vapor to dry air. Values for Cp and water vapor-dry air ratio are taken from Monteith & Unswoth (2013).

The slope of the saturation vapor pressure curve, s (units: Pa per degree K), which describes the derivative form of SVP to temperature and is determined:

s=17.38×239×SVP239+T-273.152								(3.12)

Air density must also be calculated; we use the NIST simplified air density formula with buoyancy correction (NPL 2021):

ρ=0.34844×P-RH%0.00252×T-273.15-0.020582T				(3.13)

Where P is air pressure in millibars or hPa; RH (Equation 3.8) should be expressed in percentage terms (0 ~ 100).

Users should note that the only place to use surface air pressure from coarse resolution global meteorological data GMAO/NASA is for calculation of AVP above. Except that, air pressure (in Pa) in the ET algorithm is derived from 500m SRTM elevation based on an equation for standard atmosphere (P168 in Iribane and Godson, 1981), and MOD16 and VNP16 calculates air pressure (in Pa) as a function of elevation:

Pa=Pstd×1+lzTstd5.256							(3.14)

where l is the standard temperature lapse rate (-0.0065 deg K per meter), z is the elevation in meters; Pstd and Tstd are the standard pressure (101325 Pa) and temperature (288.15 deg K) at sea level, respectively. The exponent 5.256 is a pre-determined quantity based on physical constants and derived from the discussion on atmospheric statics in Iribane & Godson (1981).  The original formular in Iribane & Godson (1981) is below.

Pa=1013.25 ×(1-2.255 ×10-5 × z)5.256 				(3.15)

### 3.1.3 Evaporation from Wet Canopy

Evaporation from a wet canopy occurs when a portion of the surface area under investigation is saturated with water; it consists of precipitated water intercepted by the vegetation canopy (LAI). As with all the components of ET, it is calculated separately for daytime and nighttime inputs. First, net radiation to the canopy is calculated as:

Acanopy=FC×A								(3.16)

Where A is calculated as in Equation 3.1. The movement of water vapor from the surface to the atmosphere is analogized to Ohm’s Law of an electrical circuit (Monteith & Unsworth, 2013; Zhang et al. 2016). Therefore, the next step involves calculating the resistances to the flow of water vapor. The resistance to radiative heat transfer through the air, rR, is given as a function of temperature (T, degrees K) and air density (ρ):

rR=ρCp4σT3									(3.17)

Where σ is the Stefan-Boltzmann constant.

The resistant of wet canopy to sensible heat is given in terms of the leaf area index (LAI), Fwet, and the leaf conductance to sensible heat (per unit LAI), gSH, which is a free parameter:

rSH=1gSH×LAI×Fwet								(3.18)

Similarly, the resistance of leaf surfaces in wet canopy to evaporated water, rWV, is given in terms of the leaf conductance to evaporated water (per unit LAI), gWV, a free parameter:

rWV=1gWV×LAI×Fwet								(3.19)

And, finally, the aerodynamic resistance to evaporated water on the wet canopy surface, rwet, is given by combining the resistances rR and rSH in parallel, because heat (of the wet canopy surface) can be lost through either channel:

rwet=rSHrRrSH+rR									(3.20)

We now have all the quantities necessary to calculate the wet canopy evaporation flux:

λEcanopy=FwetsAcanopy+ρCpVPD Fcrwets+γ rwv/rwet					(3.21)

Where A  is day or nighttime dependent for the wet canopy evaporation in day or nighttime.  Again 0.622 is the ratio of molecular weights of water vapor to dry air. Note that if the absolute value of Fwet × LAI <= 1.e-7, then λEcanopy is also zero besides the threshold of RH < 70% shown in the above Equation 3.1, indicating respective dry canopy or sparse vegetation.

### 3.1.4 Evaporation from Bare Soil Surfaces

Calculating evaporation from bare soil surfaces requires calculating both potential evaporation (PET) from the unsaturated soil surface and actual evaporation from the saturated soil surface. As with evaporation from wet canopy, we begin with calculating the resistances to water vapor fluxes.

The total aerodynamic resistance to water vapor, rtotal, is given in terms of pre-determined (calibrated) quantities including:

- rBL,max, the maximum boundary-layer resistance;

- rBL,min, the minimum boundary-layer resistance;

- VPDclose, the vapor pressure deficit (VPD) at which stomata are almost completely closed due to water stress;

- VPDopen, the VPD at which stomata are almost completely open, i.e., experiencing minimal water stress.

rtotal strongly depends on the atmospheric demand for water vapor (i.e., VPD):

rtotal=rcorrrBL,minVPD≤VPDopenrBL,max-rBL,max-rBL,minVPDclose-VPDVPDclose-VPDopenVPDopen<VPD<VPDcloserBL,maxVPD≥VPDclose	(3.22)

Essentially, when VPD is low (less than or equal to VPDopen), the boundary-layer resistance is at its maximum (rBL,max); the atmosphere’s demand for water is very low, so there is greater resistance to accepting more water vapor from the surface. When VPD is high, (greater than or equal to VPDclose), atmospheric water vapor is relatively scarce and the boundary-layer resistance is at a minimum. In between these two extremes, we linearly interpolate the boundary layer resistance.

As the conductance of water vapor through the air varies with the air’s temperature and pressure, and because prescribed values are assumed to be representative of standard temperature (293.15 deg K) and pressure (101300 Pa) conditions, a correction factor, rcorr, is applied; this is used elsewhere as well:

rcorr=101300PaT293.151.75-1							(3.23)

Aerodynamic resistance at the soil surface, rAS, is calculated as the parallel resistance of rR (Equation 3.17) and rtotal:

rAS=rRrtotalrR+rtotal									(3.24)

Potential evaporation from the soil consists of the same, core PM equation:

λEsoil*=sAsoil+ρCp1-FCVPDrAS-1s+γrtotalrAS-1							(3.25)

where Asoil is calculated as in Equation 3.2. But actual evaporation from the saturated soil surface is calculated as:

λEsoil,sat=FwetλEsoil*								(3.26)

where Fwet is calculated as in Equation 3.9. While the actual evaporation of the unsaturated soil surface is calculated by scaling the potential evaporation by an empirical soil moisture constraint:

λEunsat=1-FwetλEsoil*RH100VPDβ						(3.27)

where VPD is the vapor pressure deficit and β is a free parameter, though in MOD16 Collection 6.1 and earlier product versions its value was fixed at 250 (Pa). RH is calculated as in Equation 3.8. Because reliable good quality global soil moisture datasets were unavailable when MOD16 was first developed, this “soil moisture” constraint is based on the relative humidity (RH, 0~100).

Finally, actual evaporation from bare soil surfaces is given as the sum of the evaporation from saturated and unsaturated fractions:

λEsoil=λEsoil,sat+λEunsat							(3.28)

### 3.1.5 Canopy Transpiration

Transpiration from plants depends on a key parameter, the canopy conductance, which is derived from the mean stomatal and cuticular conductances of the various leaf elements that make up the canopy. Specifically, canopy conductance to transpired water vapor per unit LAI (CC), is derived from the parallel conductance of cuticular (gC) and stomatal (or surface) conductance, in series with the leaf boundary layer conductance (gBL):

CC=gBLgS+gCgBL+gS+gC  iff  LAI>0,1-Fwet>0					(3.29)

where gS is the surface conductance, described below. If LAI or 1-Fwet are equal to zero, then canopy conductance is also zero.

The surface conductance is a proxy for the bulk conductance of water vapor from plant stomata, throughout the canopy. It is approximated using linear functions of VPD and daily minimum temperature (Tmin):

gS=CL×fTmin×fVPD×rcorr						(3.30)

where rcorr is calculated as in Equation 3.23 and CL is the mean potential stomatal conductance per unit LAI (a free parameter) and f() represents linear functions that transform VPD or Tmin into dimensionless scalars ranging between 0,1:

fTmin=1Tmin≥Tmin,openTmin-Tmin,closeTmin,open-Tmin,closeTminin between0Tmin≤Tmin,close		  		(3.31)

fVPD=1VPD≤VPDopenVPDclose-VPDVPDclose-VPDopenVPD in between0VPD≥VPDclose				(3.32)

Although the stomata of many plant species do not entirely close at night, in VNP16, it is assumed that gS=0 during nighttime, as this optimizes the intrinsic trade-off between water loss and carbon gain during a photoperiod (night) in which carbon gain typically isn’t possible due to the lack of photosynthetically active radiation.

Leaf boundary layer conductance (gBL) is calculated as:

gBL=gSH×LAI×1-Fwet							(3.33)

where gSH is the leaf conductance to sensible heat per unit LAI, a free parameter.

Cuticular conductance is calculated based on the free parameter gcuticular, which is the expected average leaf cuticular conductance:

gC=gcuticular×rcorr								(3.34)

The last term in our plant transpiration calculation is the aerodynamic resistance of the dry canopy, rdry, calculated as a parallel resistance to radiative (rR, Equation 3.17) and convective heat transfer, where the latter is the inverse of leaf conductance to sensible heat (gSH):

rdry=gSH-1rRgSH-1+rR									(3.35)

Finally, we compute plant transpiration as:

λEtrans=1-FwetsAC+ρCpFCVPDrdry-1s+γ1+CC-1rdry-1						(3.36)

Nighttime has the same equation as the daytime except that gS = 0., i.e., stomatal is closed due to no sunlight. Also, if Fwet=1, then λEtrans=0.

## 3.2 Total Daily ET and Potential ET

Total daily evapotranspiration (ET) is the sum of the three components, canopy evaporation (λEcanopy), bare soil evaporation (λEsoil), and transpiration (λEtrans)

λE=λEcanopy+λEsoil+λEtrans						(3.37)

Daily potential ET (PET) is also calculated in VNP16; it is the sum of wet canopy evaporation, evaporation from saturated soil (λEsoil,sat), evaporation from unsaturated soil without the soil moisture correction (FwetλEsoil*), and potential transpiration, (λEtrans,potential):

λEpotential=λEcanopy+λEsoil,sat+1-FwetλEsoil*+λEtrans,potential		(3.38)

where potential transpiration is given by the Priestley-Taylor equation (Priestley and Taylor, 1072):

λEtrans,potential=αsAC1-Fwets+γ							(3.39)

where α=1.26.

Note that all ET values are given in energy units (W m-2). If you wish to obtain a water vapor mass flux (units: kg per square meter per second), then divide by the latent heat of vaporization (λ, units: J per kilogram). Because 1 kg of pure water covers a square meter to 1 mm thickness, the mass flux (kg m-2 s-1) is also equivalent to millimeters per second (mm s-1).

## 3.3 Free Parameters

Table 3.1: The free parameters (calibrated parameters) used in MOD16 and VNP16.

| Parameter | Description |
| --- | --- |
| Tmin,close | Temperature at which stomata almost completely closed (C) |
| Tmin,open | Temperature at which stomata almost fully opened (C) |
| VPDclose | VPD at which stomata are almost completely closed (Pa) |
| VPDopen | VPD at which stomata are almost completely opened (Pa) |
| gSH | Leaf conductance to sensible heat per unit LAI (m s-1 LAI-1) |
| gWV | Leaf cond. to evaporated water per unit LAI (m s-1 LAI-1) |
| gcuticular | Leaf cuticular conductance (m s-1) |
| CL | Mean potential stomatal cond. per unit leaf area (m s-1) |
| rBL,min | Minimum leaf boundary layer resistance (s m-1) |
| rBL,max | Maximum leaf boundary layer resistance (s m-1) |
| β | Soil moisture constraint on potential soil evaporation |

# 4. Operational Details and Uncertainty of VNP16

VNP16 combines operational, remotely sensed data and global, hourly meteorological data. This section discusses some of the assumptions and special issues involved in development of the input variables and their influence on the final ET estimates.

## 4.1 Dependence on MODIS Land-Cover Classification

The current C2 VNP16 product uses the same land-cover data as the operational Collection 6.1 500-m MODIS ET: a 3-year smoothed land cover data set based on the land cover type 1 classification used in the Collection 5.1 500-m MCDLCHKM (Figure 2). The importance of this product cannot be overstated as the VNP16 algorithm relies heavily on land cover type to identify ecosystem parameters from the BPLUT. The land-cover type 1 from 500-m MCDLCHKM is a 17-class IGBP (International Geosphere-Biosphere Programme) land-cover classification map (Running et al. 1994, Belward et al. 1999, Friedl et al. 2010) (Table 3.1).

Figure 2: C2 500m VIIRS ET uses the IGBP land cover type 1 classification from the 500-m Collection 5.1 MCDLCHKM dataset, which defines the following major land cover types: Evergreen Needleleaf Forest (ENF), Evergreen Broadleaf Forest (EBF), Deciduous Needleleaf Forest (DNF), Deciduous Broadleaf Forest (DBF), Mixed forests (MF), Closed Shrublands (CShrub), Open Shrublands (OShrub), Woody Savannas (WSavanna), Savannas (Savanna), grassland (Grass), and Croplands (Crop). Note that in this figure, we combined CShrub and OShrub into a general Shrub class, and WSavanna and Savannas into a general Savanna class.

Given the global nature and daily time-step of VNP16 processing, a broad classification scheme, which retains the essence of land cover, is necessary. Since Collection 2 VIIRS products are designed at a 500-m grid scale, it can be difficult to obtain accurate land cover in areas with complex vegetation; misclassification can occur (Sexton et al., 2016). However, studies have suggested that the MODIS vegetation maps are accurate to within 65-80% correctly classified, with higher accuracies for pixels that are largely homogeneous, and that they allow for consistent monitoring of global land cover (Friedl et al. 2010).

Table 4.1: The land-cover types used in the VNP16 algorithm, as derived from the IGBP Type 1 global land cover classification (Friedl et al. 2010).

| Class Value | Class Description |
| --- | --- |
| 0 | Water |
| 1 | Evergreen Needleleaf Forest |
| 2 | Evergreen Broadleaf Forest |
| 3 | Deciduous Needleleaf Forest |
| 4 | Deciduous Broadleaf Forest |
| 5 | Mixed Forest |
| 6 | Closed Shrubland |
| 7 | Open Shrubland |
| 8 | Woody Savanna |
| 9 | Savanna |
| 10 | Grassland |
| 12 | Cropland |
| 13 | Urban or Built-Up |
| 16 | Barren or Sparsely Vegetated |
| 254 | Unclassified |
| 255 | Missing Data |

## 4.2 Assumption of Constant Biome Properties

Arguably, the most significant assumption made in the VNP16 logic is that biome-specific physiological parameters do not vary across space or time. These parameters are prescribed for the different land cover types in the ancillary Biome Properties Look-up Table (BPLUT) used for VNP16 processing. The BPLUT constitutes the physiological framework for controlling simulated ET. These biome-specific properties are not differentiated within a given biome, nor are they varied at any time during the year. In other words, a semi-desert grassland in Mongolia is treated the same as a tallgrass prairie in the Midwestern United States. Likewise, a sparsely vegetated boreal evergreen needleleaf forest in Canada is functionally equivalent to its coastal temperate evergreen needleleaf forest counterpart. Spatial and temporal variation in ET within each land cover class is instead driven by associated heterogeneity in daily surface meteorology, remotely sensed vegetation canopy and soil conditions. Table 4.2 is the newly calibrated BPLUT for the future Collection 7 MODIS ET/PET and current Collection 2 VIIRS ET/PET data products.  The details on the Cal/Val of the BPLUT will be available in Endsley et al (2024, in preparation).

Table 4.2.  Biome-Property-Look-Up-Table (BPLUT) for VIIRS C2 ET/PET/LE/PLE algorithm with GMAO/NASA data and the Collection 2 VIIRS FPAR/LAI as inputs. The full names for the IGBP land cover classification system in MCDLCHKM dataset (fieldname: Land_Cover_Type_1) are, Evergreen Needleleaf Forest (ENF), Evergreen Broadleaf Forest (EBF), Deciduous Needleleaf Forest (DNF), Deciduous Broadleaf Forest (DBF), Mixed forests (MF), Closed Shrublands (CShrub), Open Shrublands (OShrub), Woody Savannas (WSavanna), Savannas (Savanna), Grassland (Grass), and Croplands (Crop). Note this BPLUT for C2 VIIRS ET/PET/LE/PLE is different from the current C6.1 MODIS one as it is an adjusted BPLUT from a recalibrated MODIS BPLUT (Endsley et al., 2024, in preparation).

| UMD_VEG_LC | ENF | EBF | DNF | DBF | MF | CShrub | OShrub | Woody Savanna | Savanna | Grass | Crop |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tmin_min (C) | -8 | -8 | -8 | -6 | -7 | -8 | -8 | -8 | -8 | -8 | -8 |
| Tmin_max (C) | 3.6 | 20.2 | 3.7 | 21.4 | 17.8 | 19.4 | 11.1 | 10.9 | 8.1 | 5 | 21.5 |
| VPD_min (Pa) | 650 | 1000 | 650 | 650 | 650 | 650 | 650 | 650 | 650 | 650 | 650 |
| VPD_max (Pa) | 4636 | 4285 | 2462 | 3434 | 3923 | 4673 | 5365 | 4367.5 | 5109 | 4156 | 4963 |
| gSH (m/s) | 0.014264 | 0.013 | 0.068 | 0.014 | 0.017 | 0.02 | 0.02 | 0.027 | 0.019 | 0.039 | 0.024 |
| gWV (m/s) | 0.014264 | 0.013 | 0.032 | 0.014 | 0.02 | 0.02 | 0.02 | 0.023 | 0.038 | 0.039 | 0.024 |
| gcuticular (m/s) | 7e-05 | 5e-05 | 7e-05 | 6e-05 | 6e-05 | 5e-05 | 5e-05 | 0.0002 | 0.00012 | 0.00023 | 0.00016 |
| CL (m/s) | 0.00408677 | 0.0021 | 0.0078 | 0.0063 | 0.0071 | 0.0137049 | 0.0162 | 0.0104 | 0.009 | 0.0115 | 0.0157 |
| rBL,min (s/m) | 222 | 503 | 464 | 511 | 277 | 200 | 231 | 506 | 82 | 319 | 194 |
| rBL,max (s/m) | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 |
|  | 250 | 250 | 250 | 250 | 250 | 250 | 250 | 250 | 250 | 250 | 250 |

## 4.3 Vegetation Properties and Albedo

The fPAR/LAI inputs used for the ET calculations are obtained from an 8-day, composite VIIRS data product, VNP15A2H. The VNP15 compositing algorithm uses a simple selection rule, whereby, the maximum fPAR (across the eight days) is chosen as the output pixel. The same day chosen to represent the fPAR measure also contributes the current pixel’s LAI value. This means that although ET is calculated daily, the VNP16 algorithm necessarily assumes that leaf area and fPAR do not vary during a given 8-day period. Compositing of LAI and fPAR is required to provide an accurate depiction of global leaf area dynamics with consideration of spectral cloud contamination, particularly in the tropics.

VIIRS ET uses 1-km shortwave black-sky broadband albedo (Albedo_BSA_shortwave) from the VIIRS M bands’ BRDF data product, VNP43MA3 (or VJ143MA3), which is a 16-day, rolling, daily albedo data product. The daily Albedo_BSA_shortwave is used as the input albedo to the 500-m ET algorithm, where the four 500-m pixels within a 1-square kilometer Albedo_BSA_shortwave pixel share the same albedo value. When the albedo value is missing (i.e., is equal to the fill value) for snow-free (snow/ice status is derived from FparExtra_QC in VNP15A2H), vegetated land pixels due to a poor retrieval, the snow-free gap-filled MODIS albedo is used (Sun et al., 2017). Otherwise, an albedo value of 0.4, a default value is used. In the future, we may create intermediate climatology/gap-filled VIIRS albedo in a fashion like how the climatology/gap-filled fPAR/LAI is created to support VIIRS (see Section 4.4).

## 4.4 Cloud/Aerosol Screening and Year-End Gap-Filling

Cloud- and/or aerosol-contaminated 8-day fPAR/LAI inputs from the 8-day VNP15A2H or VJ115A2H products can introduce substantial error to VNP16 estimated ET or PET. To address this issue, VIIRS ET and PET follow the method successfully used for C6.1 MOD16 processing. For the forward VNP16 processing, a 5-year rolling climatology fPAR/LAI is used to replace low-quality VIIRS fPAR/LAI values, labeled as such in the product quality control data fields. This process ensures a consistent, low-latency and gap-free product for operational use, although residual effects from the loss of dynamic fPAR/LAI observations can still limit product performance, particularly in persistent cloudy regions including the humid tropics and northern temperate and polar regions with a maritime climate. For higher product performance, in each early-year phase, when all 8-day VIIRS VNP15A2H or VJ115A2H of the previous year are available, a year-end, gap-filled 8-day VNP16A2GF and VNP16A3GF for the previous year is produced. Therefore, if the study period is not for the present year, we recommend using the year-end, gap-filled (GF) 8-day (A2) and annual (A3) data products.

To promote the use of the superior, gap-filled products, climatology-filled products of previous years will be progressively deprecated, i.e., there will be no forward annual VNP16A3 and VJ116A3 of the previous year, and we will stop delivery of the regular 8-day VNP16A2/VJ116A2 of the previous year in early summer (June) of the present year. This will direct users to the higher-quality gap-filled data products: 8-day VNP16A2GF/VJ116A2GF and the corresponding annual VNP16A3GF/VJ116A3GF, over all previous years.

### 4.4.1 fPAR/LAI Climatology and Forward VNP16A2

For the MODIS ET data product, starting from C6.1 MOD16, we have introduced an fPAR/LAI climatology and named it as MCD15A2HCL. The data from MCD15A2HCL are used as backup input to replace low quality (low-QC) fPAR/LAI values in each pixel in MOD15A2H or MYD15A2H for the corresponding 8-day periods. MCD15A2HCL is the average of the best fPAR/LAI value for a given vegetated pixel for the given 8-day period across the past 5 years, based on the data quality flags from both MOD15A2H and MYD15A2H. If there are no reliable fPAR/LAI data available for an 8-day period across all 5 years, the gap-filling method proposed by Zhao et al. (2005) is used. We chose a 5-year span because this is the time span for global climate fluctuations, such as the El Niño- Southern Oscillation (ENSO); and because ENSO has significant impacts on vegetation growth and other biosphere activities (Keeling et al., 1995; Nemani et al., 2003; Du et al., 2021). A longer time span may result in an unrealistic fPAR/LAI climatology because averaging obscures vegetation greenness changes caused by ecosystem disturbances/recoveries or land-use/land-cover change (Mildrexler et al., 2007; Mildrexler et al., 2009; Song et al. 2018). For C2 VNP16 and VJ116 forward data products, we still use the MODIS MCD15A2HCL as a backup fPAR/LAI source to replace those values with poor quality from VNP15A2H (or VJ115A2H) given the general cross-product consistency (Xu et al., 2018). However, we plan to use VIIRS climatology fPAR/LAI for VIIRS ET/PET data products in the future.

Figure 3: The diagram shows how fPAR from VNP15A2H and 5-year climatology MCD15A2HCL are combined to fill the gaps of contaminated fPAR in VNP15A2H using backup fPAR from MCD15A2HCL to enhance the forward VNP16 for two tiles H11V08 (Amazon) and H25V04 (Mongolia). Compared to the original fPAR from VNP15A2H, the combined fPAR climatology is more impactful for H11V08 in the cloudy Amazon region (78.2% replacement) than for H25V04 (40.9% replacement) in Mongolia.

Figure 3, using fPAR as an example, shows how the VNP16A2 process combines 8-day VNP15A2H with the corresponding 8-day climatology MCD15A2HCL to fill gaps from contaminated fPAR/LAI for ET and PET calculations. For a given vegetated pixel, if fPAR/LAI from VNP15A2H are contaminated (low QC), the corresponding climatology fPAR/LAI will be used to replace the contaminated values for the ET calculations. We use two tiles: H11V08 (in Amazon) and H25V04 (in Mongolia) as examples. Figure 3 shows how the combination of VNP15A2H and MCD15A2HCL generally results in higher fPAR values for H11V08, but only a slight increase in fPAR for H25V04. This is because the Amazon region is frequently cloud-covered, while Mongolia’s dry climate leads to clear conditions most of the time. For the Amazon tile, during this 8-day span starting on day-of-year 81 in 2018, 78.21% of vegetated pixels had low data quality and were replaced by the climatology values. In contrast, only 40.88% of vegetated pixels in the Mongolia tile are replaced during the 8-day span starting on day-of-year 177 in 2018. Currently, VNP16 uses a 5-year clear-sky climatology derived from the MODIS MCD15A2HCL record, while a VIIRS fPAR/LAI climatology is planned for use in future ET product releases.

Figure 4: Comparison of ET from C6 MOD16A2 (left panel) with that from C6.1 (right panel) for two selected tiles: H11V08 (in Amazon) and H25V04 (in Mongolia). C6.1 quality is improved because data gaps caused by cloud contamination have been filled using climatology fPAR/LAI, especially in the relatively cloudy Amazon region. Additionally, gap-filled albedo also plays a role in enhancing the ET data quality.

Figure 4 is directly copied from the MODIS ET/PET C6.1 User Guide and shows the comparison of C6 and C6.1 MOD16A2H for the same two tiles: H11V08 in Amazon and H25V04 in Mongolia. Again, the relatively humid Amazon tile has many more data gaps than the relatively dry Mongolian tile; the large white area shown in the southern half of the Mongolian tile is almost entirely due to non-vegetated land pixels rather than cloud cover. C6.1 fills many “missing” ET/PET values in C6 for cloudy Amazonian tile by using the MCD15A2HCL climatology to fill low-quality or missing fPAR/LAI values, resulting in “full,” higher-quality forward MOD16A2H data products. Moreover, starting from C6.1, the introduction of snow-free, gap-filled MODIS albedo (Sun et al., 2017) to fill missing albedo values (under poor atmospheric conditions) further enhanced the quality of MODIS forward ET/PET data products. VIIRS C2 forward ET/PET processing follows the same method: using MODIS climatology fPAR/LAI and snow-free, gap-filled MODIS albedo to replace low-quality VIIRS fPAR/LAI and albedo values for the daily ET calculations. In the future, we will use VIIRS-based climatology fPAR/LAI and snow-free, gap-filled albedo so that all dynamic remote sensing inputs will be from VIIRS.

### 4.4.2 Year-end Gap-filled VNP16A2[3]GF

At the end of each year, we address the issue of the contaminated VIIRS fPAR/LAI inputs to VNP16A2[3]GF in a more robust fashion than the climatology-filling used in the 8-day forward product (described above). First, we remove low-quality fPAR and LAI data, based on the QC label for every pixel. If any fPAR/LAI pixel did not meet the quality screening criteria, its value is determined through linear interpolation between the previous period’s value and that of the next period to pass the screening process (Zhao et a., 2005; Mu et al., 2007). As a result of this gap-filling, under most conditions, 8-day composited ET will increase because fPAR and LAI generally increase, implying more vegetation cover and more leaves.

To further demonstrate how the year-end gap-filling method improves the data quality, we again compare two tiles (Figure 5): 1) H11V08 (left sub-panel) representing an Amazon sub-region with frequent cloudiness (major land cover type is evergreen broadleaf forest); and 2) H25V04 (right sub-panel) representing a relatively clear-sky region in Mongolia (major land cover types are grassland and barren). We show only fPAR, here, because LAI has a very similar pattern. For VNP16 (Figure 6), we show only ET because changes in LE, PET and PLE have similar spatial patterns to ET.

Figure 5: Comparisons of 8-day combined fPAR of VNP15A2H and MCD15A2HCL (left panel) with the year-end gap-filled products (right panel) for two tiles: Amazonia H11V08 and Mongolia H25V04. The year-end, gap-filled fPAR/LAI record displays more realistic vegetation dynamics, especially in areas (e.g. cloudy regions) with a greater frequency of lower quality fPAR/LAI retrievals, which improves ET performance.

As shown in Figures 3 and 5, due to the frequent cloudiness in an 8-day period of 2003081, some fPAR retrievals in the Amazon tile H11V08 are contaminated with unrealistically low values; we call these errors in fPAR retrievals “gaps” in the time series for a given pixel. At the end of each year, when all 46 8-day VNP15A2H values are available for a given pixel, the year-end, interpolated fPAR/LAI values are used to fill these gaps, resulting in more reliable, gap-filled fPAR/LAI images. We named this intermediate, gap-filled fPAR/LAI product as VNP15A2HGF. When using pixels of VNP15A2HGF (VJ115A2HGF) to replace low-quality pixels in VNP15A2H (VJ115A2H) during VNP16 (VJ116) reprocessing, the result is 8-day VNP16A2GF (VJ116A2GF) and annual VNP16A3GF (VJ116A3GF). As depicted in Figures 6 and 7, coverage in both the 8-day VNP16A2GF and annual VNP16A3GF products are improved. As mentioned earlier, no forward annual A3 ET data products are produced from the VNP16A3 (VJ116A3) record and only the gap-filled annual VNP16A3GF (VJ116A3GF) product is available.

Figure 6: Comparisons of 8-day ET from the forward VNP16A2 (upper panel) and year-end gap- filled VNP16A2GF (bottom panel) for two tiles. The closeness of the two estimated results indicates the effectiveness of using an fPAR/LAI climatology as input for the forward data products. The end-of-year gap-filled fPAR/LAI (and albedo) derived ET record (GF) is expected to have even greater performance due to even more realistic vegetation dynamics.

Figure 7: Comparisons of annual total ET from the forward VNP16A3 (upper panel) and year-end, gap-filled VNP16A3GF (bottom panel) for the same two tiles shown in Fig. 6.

## 4.5 GMAO Hourly Meteorological Data

The VNP16 algorithm computes ET/PET and LE/PLE at a daily time step. This is made possible by the daily, global meteorological reanalysis data, including average and minimum air temperatures, incident downward solar radiation, and specific humidity provided by the GMAO/NASA GEOS-5 data assimilation system (Rienecker et al. 2008) at hourly time steps and with approximately 0.5-degree resolution. We aggregated the hourly meteorological data to sub-daily (day- and nighttime) and daily (24-hour) time steps for use in VNP16 processing. The GMAO/NASA GEOS-5 Forward Processing (FP-IT, https://gmao.gsfc.nasa.gov/GMAO_products/), like other global meteorological reanalyses, has uncertainties, especially for regions with poor weather station coverage and small-scale convection processes (mostly in the tropics) which cannot be sufficiently depicted by the coarse-resolution meteorological data assimilation system. Therefore, uncertainties in GEOS-5 and subsequent estimates of ET are higher in the tropics than in non-tropical regions (Zhao et al. 2006).

One major challenge is the inconsistency in spatial resolution between half-degree GEOS-5 FP-IT meteorological data and 500-m VIIRS Collection 2 pixels. To mitigate the appearance of visual artifacts introduced by the coarse-scale meteorological data, we apply spatial smoothing to the meteorological data, as proposed earlier for the MOD17 data product (Zhao et al., 2005). The spatial interpolation method is detailed below.

The four GEOS-5 cells nearest a given 500-m VIIRS pixel are used in the interpolation algorithm, which is a non-linear variation on inverse-distance approaches. We use a modified cosine function of the form below, as it constrains the output spatial weighting value to between 0 and 1:

Di=cos6π2didmaxi								(4.1)

where, Di is the non-linear distance between the 500-m VIIRS pixel and any one of four surrounding GEOS-5 cells; di is the great-circle distance between the 500-m pixel and the same GEOS-5 cell; and dmax is the great-circle distance between the two farthest GEOS-5 cells of the four being used. This ensures that Di=1 when di=0, and Di=0 when di = dmax. For an earlier version of GEOS-5 data with a much coarser resolution, we used a cosine function raised to the fourth power. We now use the power 6, as the resolution of the GEOS-5 meteorological data has increased.

Based on the non-linear distance (Di), the weighted value Wi can be expressed as:

Wi=Di∑6i=1Di									(4.2)

and therefore, for a given, the corresponding smoothed value V (i.e., interpolated Tmin, Tmean, VPD, or short-wave radiation) is:

V=i=16WiVi									(4.3)

Using the measurements of daily meteorological data from thousands of WMO weather stations as a baseline, we found that, relative to the nearest-neighbor resampling method, this spatial smoothing method resulted in approximately 75% (approximately 20%) of weather stations showing increased correlation (root mean squared error) (Zhao et al., 2005).

# 4.6 Validation of VNP16/VJ116

At the writing of this documentation, the published validation activities reflect the MODIS ET products, as the VIIRS VNP16 ET/PET data products have not yet been released. Some validation results for MODIS ET are still relevant to VIIRS ET, and users can refer to the MODIS ET/PET C6.1 User Guide and relevant publications online. Intercomparisons of the recent VIIRS ET science test with C6.1 counterparts also reveal good agreement between the two data products when the same C6.1 BPLUT were applied to both MODIS and VIIRS, as indicated by the statistical metrics in Figure 8.

Figure 8: Comparison of annual total ET for 2018 from VIIRS (VNP16A3GF and VJ16A3GF) with the MODIS C6.1 MYD16A3GF product. The spatial patterns (left panel) and one-by-one line density intercomparisons between VIIRS annual ET and MODIS C6.1 (right panel) are largely consistent. Note that in this comparison, VIIRS ET used the MODIS C6.1 BPLUT, not the newly calibrated BPLUT (Endsley et al., 2024), which is being used by the C2 VIIRS ET.

We will update this section when more research papers on the data products are available.

The current organization of VIIRS land product validation is here:

https://viirsland.gsfc.nasa.gov/

and specifics for VIIRS ET and PET can be found here:

https://viirsland.gsfc.nasa.gov/Products/NASA/ET_ESDR.html

# 5. Description of 500-m VNP16/VJ116 Data Products

VNP16A2/VJ116A2 are 8-day forward data products, where only the data for the current year are publicly available; these data are considered suitable for operational applications where a low product latency is needed. For greater precision, users are encouraged to use the reprocessed, year-end, gap-filled 8-day VNP16A2GF/VJ116A2GF and annual VNP16A3GF/VJ116A3GF ET products. As detailed in the above Section 4.4.2, users should use the higher quality gap filled (GF) data products if the study data period doesn’t include the present year. Users may mix present year forward data products with the gap-filled from the previous years as Figures 6 and 7 above have shown the closeness of the two, but caution should be taken because of inconsistency.

Please note that to avoid duplication, the descriptive information mainly focuses on VNP16. The information for VNP16 applies to VJ116 unless specified otherwise.

VIIRS 16 data products are stored in gridded HDFEOS5 format, a self-described scientific data file format (https://www.hdfeos.org/). Users may use a free software, HDFVIEW (https://www.hdfgroup.org/tag/hdfview-release/), to check the file’s global attributes, attributes of different data fields, and the data to gain more information about the data products. As a level-4 VIIRS land data product, each HDFEOS5 file stores the data over a 10-degree tile domain in Sinusoidal map projection. For details on the 10-degree tile system, please visit the following LP-DAAC website:

https://lpdaac.usgs.gov/data/get-started-data/collection-overview/missions/s-npp-nasa-viirs-overview/#viirs-grid-systems

Below we provide key information on the data products to help users utilize the data.

## 5.1 File Naming Conventions

VIIRS filenames follow a naming convention which gives useful information regarding the specific product. Product short name designations are as follows: VNP for Suomi-NPP, and VJ1 for Joint Polar Satellite System-1 (JPSS-1) or NOAA-20.

In this example of an 8-day Gap-Filled VNP16A2GF product, the filename VNP16A2GF.A2018193.h11v08.002.2024262234953.h5 indicates:

- VNP16A2GF - Product Short Name for the 8-day Gap-Filled VNP ET/PET/LE/PLE

- A2018193 - Julian Date of the Start Date of the 8-day Period (AYYYYDDD)

- h11v08 – 10-Degree Tile Location (hxxvxx)

- 002 - Collection Version

- 2024262234953 - Julian Date of File Creation (YYYYDDDHHMMSS)

- .h5 - Data Format (HDF-EOS5)

In another example of an 8-day forward VJ116A2 product, the VJ116A2.A2018185.h10v05.002.2024262234952.h5 denotes:

- VJ116A2 - Product Short Name for the 8-day forward VJ1 ET/PET/LE/PLE

- A2018185 - Julian Date of the Start Date of the 8-day Period (AYYYYDDD)

- h10v05 – 10-Degree Tile Location (hxxvxx)

- 002 - Collection Version

- 2024262234952 - Julian Date of File Creation (YYYYDDDHHMMSS)

- .h5 - Data Format (HDF-EOS5)

In another example of an annual Gap-Filled VJ116A3GF product, filename VJ116A3GF.A2018001.h09v05.002. 2024262235002.h5 suggests:

- VJ116A3GF - Product Short Name for the Gap-Filled Annual VJ1 ET/PET/LE/PLE

- A2018001 - Julian Date of the Start Date of the Annual Period (AYYYYDDD)

- h09v05 – 10-Degree Tile Location (hxxvxx)

- 002 - Collection Version

- 2024262235002 - Julian Date of File Creation (YYYYDDDHHMMSS)

- .h5 - Data Format (HDF-EOS5)

## 5.2 Downloading VNP16 Datasets

All MODIS land data products are archived and distributed to global users from the USGS Land Processes Distributed Active Archive Center (USGS LP DAAC), found here:

https://lpdaac.usgs.gov/

Users are encouraged to explore the method to order and download data from the LP DAAC or the more comprehensive NASA Earthdata Search engine (https://search.earthdata.nasa.gov/search). A quick way to check MODIS or VIIRS data is through USGS data pool site:

https://lpdaac.usgs.gov/tools/data-pool/

## 5.3 Description of VNP16 Datasets

### 5.3.1 8-day VNP16A2 (or VNP16A2GF)

Table 5.1 lists science data sets from the 8-day VNP16A2 and VNP16A2GF records. ET_500m and potential ET (PET), PET_500m, are the summation of 8-day total water loss through ET, whereas the associated latent heat flux and its potential, LE_500m and PLE_500m, are the average total energy over a unit area for a unit day during the composite 8-day period. Please note that the last 8-day (VNP16A2.A20??361.*.h5df) of each year is not an 8-day span but either 6 (for a leap year) or 5 days (any other year).

As listed in Table 3, for valid data (Valid_data with the valid range) of VNP16A2 (or VNP16A2GF), the real value (Real_value) of each data set (ET, LE, PET or PLE) in the corresponding units (kg m-2 per 8 d or MJ m-2 per d) can be calculated using the following equation:

Real value=Valid_data×Scale_factor					(5.1)

Table 5.1: Detailed information on science datasets in VNP16A2 and VNP16A2GF

| Data Sets | Meaning | Units | Data Type | Valid Range | Scale Factor |
| --- | --- | --- | --- | --- | --- |
| ET_500m | 8-day total ET | kg/m2/8d | int16 | -32768 ~ 32700 | 0.1 |
| LE_500m | 8-day average LE | MJ/m2/d | int16 | -32768 ~ 32700 | 0.01 |
| PET_500m | 8-day total PET | kg/m2/8d | int16 | -32768 ~ 32700 | 0.1 |
| PLE_500m | 8-day average PLE | MJ/m2/d | int16 | -32768 ~ 32700 | 0.01 |
| ET_QC_500m | Quality Control | none | uint8 | 0 ~ 254 | none |

All data sets in VNP16A2 (or VNP16A2GF), except the Quality Control (QC) field, ET_QC_500m, have valid data values ranging from -32768 to 32700 and are stored as signed 2-byte short integers (int16). Though data attributes list just one _FillValue: 32767 in the header file of VNP16A2 (or VNP16A2GF), there are, in fact, 7 fill values (listed below) for non-vegetated pixels. ET is not calculated for pixels with the following fill values:

32767 = _Fillvalue

32766 = land cover assigned as perennial salt or Water bodies

32765 = land cover assigned as barren, sparse veg (rock, tundra, desert)

32764 = land cover assigned as perennial snow, ice.

32763 = land cover assigned as "permanent" wetlands/inundated marshland

32762 = land cover assigned as urban/built-up

32761 = land cover assigned as "unclassified" or (not able to determine)

32767 = _Fillvalue

Note: two fill values, 251 and 252, in the upstream VIIRS FPAR/LAI inputs to the ET algorithm are mislabeled as 253; as a result, the subsequent VIIRS 8-day ET_500m, LE_500m, PET_500m and PLE_500m products also have two mislabeled fill values (32763 and 32764 mislabeled as 32765) because of data dependence. The fill values for those datasets are:

32766 = land cover assigned as perennial salt or Water bodies

32765 = land cover assigned as barren, sparse veg (rock, tundra, desert)

32764 = land cover assigned as perennial snow, ice.

32763 = land cover assigned as "permanent" wetlands/inundated marshland

32762 = land cover assigned as urban/built-up

32761 = land cover assigned as "unclassified" or (not able to determine)

The QC data layer, ET_QC_500m, consists of bitflags from two QC data fields, FparLai_QC and FparExtra_QC, from the upstream VNP15A2H of the corresponding 8-day period. Detailed bitflag information on those two QC data fields is below.

Table 5.2: Explanation of QC bit flags used in the ET_QC_500m dataset.

| Bit Position(s) | Description |
| --- | --- |
| 0-2: Algorithm path? | 000 = 0 Main (RT) algorithm used, best result possible |
|  | 001 = 1 Main (ET) algorithm used, saturation occurred. Good, very usable. |
|  | 010 = 2 Main algorithm failed due to bad geometry, empirical algorithm used |
|  | 011 = 3 Main algorithm failed due to problems other than geometry, empirical algorithm used |
|  | 100 = 4 Pixel not produced at all, value couldn’t be retrieved (possible reasons: bad L1B data, unusable VNP09GA data, water pixel) |
| ‍3-3: Dead detector? | 0 = Both red and NIR detectors are fine |
| ‍ | 1 = At least one band has dead detector |
| ‍4-5: Cloud detection and confidence? | 00 = 0 Confident clear |
| ‍ | 01 = 1 Probably clear |
| ‍ | 10 = 2 Problably cloudy |
| ‍ | 11 = 3 Confident cloudy |
| ‍6-6: Cloud shadow? | 0 = 0 No cloud shadow |
| ‍ | 1 = 1 Shadow |
| ‍7-7 Thin cirrus? | 0 = 0 No thin cirrus |
| ‍ | 1 = 1 Thin cirrus |

The above “Main (RT) algorithm” or “empirical algorithm” refers to the algorithm of VIIRS FPAR/LAI data products, VNP15A2H and VJ115A2H), not the algorithm of ET/PET.

For C2, NASA’s forward VNP16A2 and gap-filled VNP16A2GF, the ET_QC_500m information can provide users the quality of the original VIIRS 8-day FPAR/LAI data, input to VNP16A2[GF]. Compared to the very early stage of MOD16 data products, the usefulness of ET_QC_500m has been greatly reduced in that the original fPAR/LAI values labeled with poor quality (mostly due to cloud contamination) have been replaced either by climatology fPAR/LAI in the forward VNP16A2 production or by gap-filled fPAR/LAI in the year-end gap-filled VNP16A2GF production, as detailed in the section 4.4 above.

### 5.3.2 Annual VNP16A3GF/VJ116A3GF

Table 5.3 lists the annual VNP16A3 (or VNP16A3GF) datasets. ET_500m and PET_500m are the summation of total daily ET/PET through the year whereas LE and PLE are the corresponding average total latent energy over a unit area for a unit day through the year. LE_500m and PLE_500m have the same data type (signed 2-byte short int16), valid range, and fill values as those listed above for the 8-day VNP16A2; whereas annual ET_500m and PET_500m are saved in unsigned 2-byte short integer (uint16) format with valid range from 0 to 65500.

Similar to VNP16A2 (or VNP16A2GF), as listed in Table 5.2, for valid data (Valid_data within the valid range) of VNP16A3 (or MOD16A3GF), the real value (Real_value) of each data set (ET, LE, PET or PLE) in the corresponding units (kg m-2 yr-1 or MJ m-2 d-1) can be calculated using Equation 5.1 (above).

Table 5.3: Detailed information on science datasets in VNP16A3 and VNP16A3GF.

| Data Sets | Meaning | Units | Data Type | Valid Range | Scale Factor |
| --- | --- | --- | --- | --- | --- |
| ET_500m | annual sum ET | kg/m2/yr | uint16 | 0 ~ 65500 | 0.1 |
| LE_500m | annual average LE | MJ/m2/d | int16 | 0 ~ 32700 | 0.01 |
| PET_500m | annual sum PET | kg/m2/yr | uint16 | 0 ~ 65500 | 0.1 |
| PLE_500m | annual average PLE | MJ/m2/d | int16 | 0 ~ 32700 | 0.01 |
| ET_QC_500m | Quality Assessment | Percent (%) | uint8 | 0 ~ 100 | 1.0 |

Though data attributes list one _FillValue: 65535 for ET_500m and PET_500m with unsigned uint16 in the HDFEOS5 VNP16A3 (or VNP16A3GF) file, there are, in fact, 7 fill values as listed below for non-vegetated pixels without ET calculations.

65535 = _Fillvalue

65534 = land cover assigned as perennial salt or Water bodies

65533 = land cover assigned as barren, sparse veg (rock, tundra, desert)

65532 = land cover assigned as perennial snow, ice.

65531 = land cover assigned as "permanent" wetlands/inundated marshland

65530 = land cover assigned as urban/built-up

65529 = land cover assigned as "unclassified" or (not able to determine)

For LE_500m and PLE_500m in signed int16, similarly, though data attributes list one _FillValue: 32767 for LE_500m and PLE_500m with signed int16 in the HDFEOS VNP16A3GF file, there are, in fact, 7 fill values as listed below for non-vegetated pixels without ET calculations.

32767 = _Fillvalue

32766 = land cover assigned as perennial salt or Water bodies

32765 = land cover assigned as barren, sparse veg (rock, tundra, desert)

32764 = land cover assigned as perennial snow, ice.

32763 = land cover assigned as "permanent" wetlands/inundated marshland

32762 = land cover assigned as urban/built-up

32761 = land cover assigned as "unclassified" or (not able to determine)

Note: two fill values, 251 and 252, in the VIIRS FPAR/LAI are mislabeled as 253, and as a result, the subsequent VIIRS annual ET_500m and PET_500m (LE_500m and PLE_500m) mislabeled two fill values, 65531 (32763) and 65532 (32764), as 655333 (32765) because of data dependence.

65535 = _Fillvalue

65534 = land cover assigned as perennial salt or Water bodies

65533 = land cover assigned as barren,sparse veg (rock, tundra, desert)

65532 = land cover assigned as perennial snow, ice.

65531 = land cover assigned as "permanent" wetlands/inundated marshland

65530 = land cover assigned as urban/built-up

65529 = land cover assigned as "unclassified" or (not able to determine)

The QC data field in the annual VNP16A3GF, ET_QC_500m, is different from most VIIRS/MODIS QC data sets because it is not a series of bit flags but a more meaningful QC assessment for annual composite values. We used the method proposed by Zhao et al. (2005) to define annual ET QC as:

ET_QC_500m=100×NgNtotal						(5.2)

where Ng is the number of days during the growing season with poor-quality VIIRS/MODIS 500m LAI inputs. Ntotal is total number of days in the growing season. The growing season is defined as all days with Tmin above the value where stomata close, as in the BPLUT. For example, an ET_QC_500m value of 85 for a pixel implies that during the growing days, for 85 percent of days, gap-filled input fPAR/LAI had to be used because of unfavorable atmospheric conditions. As expected, more humid forested regions tend to have high ET_QC_500m, whereas dry regions with grasslands or shrubs tend to have low ET_QC_500m (Figure 6c in Zhao et al., 2005).

The data type of ET_QC_500m is unsigned 1-byte integer (uint8) with a valid range from 0 to 100.  Though data attributes list only one _FillValue: 255 in the HDFEOS VNPMOD16A3 (or MOD16A3GF) file, there are, in fact, 7 fill values as listed below for non-vegetated pixels.

255 = _Fillvalue

254 = land cover assigned as perennial salt or Water bodies

253 = land cover assigned as barren,sparse veg (rock, tundra, desert)

252 = land cover assigned as perennial snow, ice.

251 = land cover assigned as "permanent" wetlands/inundated marshland

250 = land cover assigned as urban/built-up

249 = land cover assigned as "unclassified" or not able to determine

# 6. List of NTSG Co-Authored Papers Using MOD16

Cleugh, H. A., Leuning, R., Mu, Q., and Running, S. W. (2007). Regional evaporation estimates from flux tower and MODIS satellite data. Remote Sensing of Environment, 106(3), 285-304.

Mu, Q., Heinsch, F. A., Zhao, M., and Running, S. W. (2007). Development of a global evapotranspiration algorithm based on MODIS and global meteorology data. Remote Sensing of Environment, 111(4), 519-536.

Renzullo, L. J., Barrett, D. J., Marks, A. S., Hill, M. J., Guerschman, J. P., Mu, Q., and Running, S. W. (2008). Multi-sensor model-data fusion for estimation of hydrologic and energy flux parameters. Remote Sensing of Environment, 112(4), 1306-1319.

Mu, Q., Jones, L. A., Kimball, J. S., McDonald, K. C., and Running, S. W. (2009). Satellite assessment of land surface evapotranspiration for the pan‐Arctic domain. Water Resources Research, 45(9).

Zhang, K., Kimball, J. S., Mu, Q., Jones, L. A., Goetz, S. J., and Running, S. W. (2009). Satellite based analysis of northern ET trends and associated changes in the regional water balance from 1983 to 2005. Journal of Hydrology, 379(1), 92-110.

Jung, M., Reichstein, M., Ciais, P., Seneviratne, S. I., Sheffield, J., Goulden, M. L., ... and Dolman, A. J. (2010). Recent decline in the global land evapotranspiration trend due to limited moisture supply. Nature, 467(7318), 951-954.

Mu, Q., Zhao, M., and Running, S. W. (2011). Improvements to a MODIS global terrestrial evapotranspiration algorithm. Remote Sensing of Environment, 115(8), 1781-1800.

Mu, Q., Zhao, M., and Running, S. W. (2011). Evolution of hydrological and carbon cycles under a changing climate. Hydrological Processes, 25(26), 4093-4102.

Mu, Q., Zhao, M., & Running, S. W. (2012). Remote Sensing and Modeling of Global Evapotranspiration. In Multiscale Hydrologic Remote Sensing: Perspectives and Applications (pp. 443-480), edited by Chang, N. B., Hong, Y., CRC Press.

Mu, Q., Zhao M., Kimball J. S., McDowell N., and Running S. W., (2013) A Remotely Sensed Global Terrestrial Drought Severity Index, Bulletin of the American Meteorological Society, 94: 83–98.

Ruhoff, A. L., Paz, A. R., Aragao, L. E. O. C., Mu, Q., Malhi, Y., Collischonn, W., ... and Running, S. W. (2013). Assessment of the MODIS global evapotranspiration algorithm using eddy covariance measurements and hydrological modelling in the Rio Grande basin. Hydrological Sciences Journal, 58(8), 1658-1676.

Smettem, K. R., Waring, R. H., Callow, J. N., Wilson, M., and Mu, Q. (2013). Satellite‐derived estimates of forest leaf area index in southwest Western Australia are not tightly coupled to interannual variations in rainfall: implications for groundwater decline in a drying climate. Global Change Biology, 19(8), 2401-2412.

Yao, Y., Liang, S., Li, X., Hong, Y., Fisher, J. B., Zhang, N., ... and Jiang, B. (2014). Bayesian multimodel estimation of global terrestrial latent heat flux from eddy covariance, meteorological, and satellite observations. Journal of Geophysical Research: Atmospheres, 119(8), 4521-4545.

Chen, Y., J. Xia, S. Liang, J. Feng, J. B. Fisher, X. Li, X. Li et al. (2014) Comparison of satellite-based evapotranspiration models over terrestrial ecosystems in China. Remote Sensing of Environment 140 (2014): 279-293.

Wang, S., M. Pan, Q. Mu, X. Shi, J. Mao, C. Brümmer, ... and T. A. Black (2015). Comparing Evapotranspiration from Eddy Covariance Measurements, Water Budgets, Remote Sensing, and Land Surface Models over Canada. Journal of Hydrometeorology, 16(4), 1540-1560.

Li, Y., M. Zhao, S. Motesharrei, Q. Mu, E. Kalnay, & S. Li (2015). Local cooling and warming effects of forests based on satellite observations. Nature Communications, 6(1), 6603.

Xia, Y., M. T. Hobbins, Q. Mu, & M. B. Ek (2015). Evaluation of NLDAS‐2 evapotranspiration against tower flux site observations. Hydrological Processes, 29(7), 1757-1771.

Yao, Y., S. Liang, X. Li, J. Chen, K. Wang, K. Jia, ... & O. Roupsard (2015). A satellite-based hybrid algorithm to determine the Priestley–Taylor parameter for global terrestrial latent heat flux estimation across multiple biomes. Remote Sensing of Environment, 165, 216-233.

Michel, D., Jiménez, C., Miralles, D. G., Jung, M., Hirschi, M., Ershadi, A., ... & Fernández-Prieto, D. (2016). The WACMOS-ET project–Part 1: Tower-scale evaluation of four remote-sensing-based evapotranspiration algorithms. Hydrology and Earth System Sciences, 20(2), 803-822.

Miralles, D. G., Jiménez, C., Jung, M., Michel, D., Ershadi, A., McCabe, M. F., ... & Fernández-Prieto, D. (2016). The WACMOS-ET project–Part 2: Evaluation of global terrestrial evaporation data sets. Hydrology and Earth System Sciences, 20(2), 823-842.

Zhang, J., Mu, Q., & Huang, J. (2016). Assessing the remotely sensed Drought Severity Index for agricultural drought monitoring and impact analysis in North China. Ecological Indicators, 63, 296-309.

Zhang, K., J. S. Kimball, and S. W. Running. 2016. A review of remote sensing based actual evapotranspiration estimation. Wiley Interdisciplinary Reviews: Water 3 (6):834–853.

Li, Y., Zhao, M., Mildrexler, D. J., Motesharrei, S., Mu, Q., Kalnay, E., ... & Wang, K. (2016). Potential and actual impacts of deforestation and afforestation on land surface temperature. Journal of Geophysical Research: Atmospheres, 121(24), 14-372.

Choi, M., M. Mu, K. Kim, K. Hwang, & J. Hur (2017). Ecosystem-dynamics link to hydrologic variations for different land-cover types. Terrestrial Atmospheric and Oceanic Sciences, 28(3), 437-462.

Yao, Y., S. Liang, X. Li, J. Chen, S. Liu, K. Jia, ... & O. Roupsard (2017). Improving global terrestrial evapotranspiration estimation using support vector machine by integrating three process-based algorithms. Agricultural and Forest Meteorology, 242, 55-74.

Román, M. O. et al. (2024) Continuity between NASA MODIS Collection 6.1 and VIIRS Collection 2 land products. Remote Sensing of Environment 302: 113963.

Endsley, K. A., M. Zhao, J. S. Kimball, T. Albrethsen, S. Devadiga (2024) Improved global estimates of terrestrial evapotranspiration using the MODIS and VIIRS sensors. Journal of Hydrometeorology, (in preparation).

# 7. References

Abbott, P. F., and R. C. Tabony (1985). The estimation of humidity parameters. Meteorological Magazine 114: 49-56.

Alduchov, O. A., and R. E. Eskridge (1996). Improved Magnus form approximation of saturation vapor pressure. Journal of Applied Meteorology (1988-2005). 1996: 601-609.

Belward, A. S., Estes, J. E., and Kline, K. D. 1999. The IGBP-DIS global 1-km land-cover data set DISCover: A project overview. Photogrammetric Engineering and Remote Sensing. 65(9):1013-1020.

Cleugh, H. A., Leuning, R., Mu, Q., and Running, S. W. 2007. Regional evaporation estimates from flux tower and MODIS satellite data. Remote Sensing of Environment. 106(3):285-304.

Du, J., J. S. Kimball, J. Sheffield, I. Velicogna, M. Zhao, M. Pan, C. K. Fisher, H. E. Beck, J. D. Watts, G. A, and E. F. Wood. 2021. Synergistic Satellite Assessment of Global Vegetation Health in Relation to ENSO‐Induced Droughts and Pluvials. Journal of Geophysical Research: Biogeosciences 126 (5).

Endsley, K. A., M. Zhao, J. S. Kimball, T. Albrethsen, S. Devadiga (2024) Improved global estimates of terrestrial evapotranspiration using the MODIS and VIIRS sensors. Journal of Hydrometeorology, (in preparation).

Fisher, J. B., Tu, K., and Baldocchi, D. D. 2008. Global estimates of the land atmosphere water flux based on monthly AVHRR and ISLSCP-II data, validated at FLUXNET sites. Remote Sensing of Environment. 112(3):901−919.

Friedl, M. A., Sulla-Menashe, D., Tan, B., Schneider, A., Ramankutty, N., Sibley, A., and Huang, X. 2010. MODIS Collection 5 global land cover: Algorithm refinements and characterization of new datasets. Remote Sensing of Environment. 114(1):168-182.

Gates, D. M. 1980. Biophysical Ecology. Springer Advanced Texts in Life Sciences. Springer New York, NY.

Iribane, J.V., and W.L. Godson, 1981. Atmospheric Thermodynamics. 2nd Edition. D. Reidel Publishing Company, Dordrecht, The Netherlands.

Jacobsen, A. and B. U. Hansen. 1999. Estimation of the soil heat flux/net radiation ratio based on spectral vegetation indexes in high-latitude Arctic areas. International Journal of Remote Sensing. 20(2):445-461.

Jarvis, P. G. 1976. The interpretation of the variations in leaf water potential and stomatal conductance found in canopies in the field. Philosophical Transactions of the Royal Society of London B: Biological Sciences. 273(927):593-610.

Keeling, C. D., Whorf, T. P., Wahlen, M., and Van der Plichtt, J. 1995. Interannual extremes in the rate of rise of atmospheric carbon dioxide since 1980. Nature. 375(6533):666-670.

Los, S. O., Pollack, N. H., Parris, M. T., Collatz, G. J., Tucker, C. J., Sellers, P. J., ... and Dazlich, D. A. 2000. A global 9-yr biophysical land surface dataset from NOAA AVHRR data. Journal of Hydrometeorology. 1(2):183-199.

Maidment, D. 1969. Handbook of Hydrology.

Mildrexler, D. J., M. Zhao, F. A. Heinsch, S. W. Running. 2007. A new satellite-based methodology for continental-scale disturbance detection. Ecological Applications. 17:235-250.

Mildrexler, D. J., M. Zhao, S. W. Running. 2009. Testing a MODIS Global Disturbance Index across North America. Remote Sensing of Environment. 113:2103-2117.

Monteith, J. L. 1965. Evaporation and environment. In Symposia of the Society for Experimental Biology (Vol. 19, p. 205).

Monteith, J. L., and M. Unsworth. 2013. Principles of Environmental Physics. Fourth Ed.

Mu, Q., Heinsch, F. A., Zhao, M., and Running, S. W. 2007. Development of a global evapotranspiration algorithm based on MODIS and global meteorology data. Remote Sensing of Environment. 111(4):519-536.

Mu, Q., M. Zhao, and S. W. Running. 2011. Improvements to a MODIS global terrestrial evapotranspiration algorithm. Remote Sensing of Environment 115 (8):1781–1800.

National Physical Laboratory. 2021. “Buoyancy Correction and Air Density Measurement.” http://resource.npl.co.uk/docs/science_technology/mass_force_pressure/clubs_groups/instmc_weighing_panel/buoycornote.pdf Accessed: September 3, 2023.

Nemani, R. R., C. D. Keeling, H. Hashimoto, W. M. Jolly, S. C. Piper, C. J. Tucker, R. B. Myneni and S. W. Running. 2003. Climate-driven increases in global terrestrial net primary production from 1982 to 1999. Science. 300:1560-1563.

Priestley, C.H.B., and R.J. Taylor, 1972. On the assessment of surface heat flux and evaporation using large-scale parameters. Monthly Weather Review, 100, 2, 81-92.

Running, S. W., Loveland, T. R., and Pierce, L. L. 1994. A vegetation classification logic based on remote sensing for use in global scale biogeochemical models. Ambio. 23:77-81.

Sexton, J. O., Noojipady, P., Song, X. P., Feng, M., Song, D. X., Kim, D. H., ... and Townshend, J. R. 2016. Conservation policy and the measurement of forests. Nature Climate Change. 6(2):192-196.

Shuttleworth, W. J., and Wallace, J. S. 1985. Evaporation from sparse crops-an energy combination theory. Quarterly Journal of the Royal Meteorological Society. 111(469):839-855.

Song, X. P., Hansen, M. C., Stehman, S. V., Potapov, P. V., Tyukavina, A., Vermote, E. F., and Townshend, J. R. 2018. Global land change from 1982 to 2016. Nature. 560(7720):639-643.

Sun, Q., Z. Wang, Z. Li, A. Erb, and C. L. B. Schaaf. 2017. Evaluation of the global MODIS 30 arc-second spatially and temporally complete snow-free land surface albedo and reflectance anisotropy dataset. Intl. Journal of Applied Earth Observation and Geoinformation. 58:36-49.

Tuzet, A., Perrier, A., and Leuning, R. 2003. Stomatal control of photosynthesis and transpiration: Results from a soil–plant–atmosphere continuum model. Plant, Cell and Environment. 26:1097-1116.

Wan, Z., Zhang, Y., Zhang, Q., and Li, Z. L. 2002. Validation of the land-surface temperature products retrieved from Terra Moderate Resolution Imaging Spectroradiometer data. Remote Sensing of Environment. 83(1):163-180.

Xu, B., T. Park, K. Yan, C. Chen, Y. Zeng, W. Song, G. Yin, Q. Liu, Y. Knyazikhin, and R.B. Myneni, 2018. Analysis of global LAI/FPAR products from VIIRS and MODIS sensors for spatio-temporal consistency and uncertainty from 2012-2016. Forests 9, 2, 73, DOI:10.3390/f9020073.

Zhang, K., J. S. Kimball, and S. W. Running. 2016. A review of remote sensing based actual evapotranspiration estimation. Wiley Interdisciplinary Reviews: Water 3 (6):834–853.

Zhao, M., Heinsch, F. A., Nemani, R. R., and Running, S. W. 2005. Improvements of the MODIS terrestrial gross and net primary production global data set. Remote Sensing of Environment. 95(2):164–176.