MTCLIM FILE DOCUMENTATION

8/1/89

Steven W. Running/Joseph C. Coughlan

University of Montana

This documentation package explains the execution and files needed for

MTCLIM, a model for extrapolation of meteorological data in mountainous

terrain from basic climatological principles.  It is designed to complement

Running et al., 1987, the original publication of this model.  There will

be a user's manual printed by the Forest Service sometime early in 1990.

Five files are included in this documentation:

MTCLIM.EXE This is the main program, compiled in Microsoft FORTRAN 4.1,

ready to execute.  Requires a 80x87 coprocessor.

MTCLIM.FOR This is the source code, in FORTRAN77. Written in MicroSoft

FORTRAN (v 4.1), however it is compatible with most compilers.  It is

written to be readable and not efficient.  See the source code for details

on methods for optimizing the model for speed.

MISS84.DAT Initial input details, including site-specific information, name

of climate file, etc.  This file is self-explanatory and should be viewed

before running the program.  Please note that it is possible to use either

metric or english units.  Be consistent!

MISS84.MTC The base station climate file, containing yearday, max temp, min

temp, dew point, and precipitation for each day.  (Note that MISS84.DAT

asks whether or not dew point is supplied - in this file it is not, so the

flag is set to "N".  If you wish to run MTCLIM with dew pt.  information,

you must create a new file with this column, then set flag to "N".  If dew

point is not supplied, it is assumed equal to night minimum temperature).

(See description of input format below)

The model itself is based on metric units.  English units are

converted inside the program before any computations are performed.  Please

make note of this if you are going to extract code from the model.

The met file can begin on day 1 and end day 365 or any period in

between, i.e. day 150 to 283.  It cannot operate from day 250 to day 100

(across day 365).  Also, those of you 'down under' will need to correct the

sun angle geometry for the southern hemisphere.

MISS84.CLM Climatic output file for the information provided in

MISS84.DAT.  For sample runs, you may change any of the switches in

MISS84.DAT and create a new output file (specify new output name if you

wish to keep both for comparison.)  The format of the output file will vary

depending on the switches you set in the .DAT file.

EXECUTION

To execute, just type MTCLIM at the Dos prompt.  The output file is

created with the name specified in MISS84.DAT.  The model will echo a

counter to the screen for each day.  The program is math intensive and can

take several minutes on a IBM AT class machine with an 80287.

If you are going to use MTCLIM output in conjunction with FOREST-BGC

(Running and Coughlan, 1988)**, you must delete the header lines in the

output file for compatibility with FOREST-BGC's read format.  Currently we

have added the .DAT file to the output file as a heading for distinguishing

model runs (See FOREST-BGC for specific input requirements).

*Running, S.W., R.R. Nemani, and R.D. Hungerford, 1987 Extrapolation of

synoptic meteorological data in mountainous terrain and its use for

simulating forest evapotranspiration and photosynthesis.  Canadian Journal

of Forest Research 17: 472-83.

**Running, S.W. and J.C. Coughlan, 1988 A general model of forest ecosystem

processes for regional applications. I. Hydrologic balance, canopy gas

exchange and primary production. Ecological Modelling 42: 125-54.

MTCLIM Input Formats

Free format FORTRAN READs of each daily input record (no commas or fixed

fields) for the following fields in the following order:

YEAR_DAY  MAX_TEMP  MIN_TEMP  DEW_PT_TEMP  PPT_1  PPT_2

Field data types:

INTEGER: YEAR_DAY (No decimal point allowed)

REAL: MAX_TEMP MIN_TEMP DEW_PT_TEMP PPT_1 PPT_2 (Decimal point allowed)

The following fields are optional and can be omitted from the input file

provided that the .DAT file is configured for the new format.  If an

optional field is missing then the input file is missing that column, all

other columns remain in the same order:

DEW_PT_TEMP  PPT_2